Table of Contents

Phase #1: Onboarding	2

Scope & Assumptions	2

Definitions	2

Identify Customers / Sites / Systems	3

Initiate Authorization Process	3

Resolve Authorization Conflicts with Customer	4

Phase #2: Configuration	5

Enrollment Conflict Resolution:	5

Program Nomination Confirmation	5

Meter Configuration Validation	5

Configuration Readiness State	5

Phase #3: Dispatch	6

Dispatch Intake	6

Dispatch Target Allocation	6

Customer Dispatch Notification	6

Load Curtailment	6

Dispatch Completion	7

Phase #4: Analyze	7

Dispatch Performance Evaluation	7

Participation Validation	7

Aggregated Performance Evaluation	7

Performance Attribution	7

Analysis Output State	7

Underperformance Handling	7

Phase #5: Get Paid	8

SkyportEnergy is the Daikin-owned customer application used to onboard customers and enable participation in energy programs.

VPP  Overview

- Leap dispatches a fleet-level performance requirement.

- SkyportEnergy distributes the dispatch across eligible FIT systems and executes control actions.

- SkyportEnergy aggregates system-level responses to evaluate fleet performance.

- Validated performance outcomes flow downstream to settlement and payment processes.

# Phase #1: Onboarding

Goal: Establish a trusted system of record by identifying eligible systems, associating them to a Site, and granting customer access required for authorization, configuration, dispatch, and performance tracking.

### Scope & Assumptions

- Eligible system: Daikin FIT

- Daikin owns:

  - Customer relationship

  - System relationship

  - Onboarding and authorization experience

- Utilities and aggregators (e.g., Leap) are upstream dependencies, not customer-facing systems

### Definitions

- Site: Physical service address that maps to one utility service location. A Site can contain multiple Daikin FIT systems.

- FIT System: Unit enrolled for dispatch and performance; Includes:

  - Outdoor unit

  - Indoor unit(s)

  - Exactly one Daikin One thermostat

- FIT System ID:  Unique ID that is created by SkyportEnergy when a Daikin One thermostat is associated with a FIT system and will be used for the lifetime of the system installation. The FIT System ID is independent of:

  - Utility

  - VPP program

  - Leap

  - Enrollment status

- Site ID: Unique ID that is created by SkyportEnergy during onboarding and will be used for the lifetime of the physical address. The Site ID is independent of:

  - Utility

  - VPP program

  - Leap

  - Enrollment status

### Identify Customers / Sites / Systems

#### Customer Identity Management

- Create a SkyportEnergy Customer record.

- Customer identity shall be based on:

  - SkyportHome account (email / login): App-based identification

  - Daikin One thermostat ownership: Validation via customer-facing physical device.

- Customer identity shall be independent of utility accounts.

#### Site Identification

- Create a Site representing a physical installation address

- Each Site:

  - Represents one utility service location

  - May contain one or more FIT systems

  - Is uniquely identifiable within SkyportEnergy

- A Customer may have multiple Sites (e.g., primary home, second home)

#### FIT System Discovery & Confirmation

- Discover all FIT systems at a Site using Daikin One thermostat associations

- Customers shall:

  - View all detected FIT systems at the Site

  - Explicitly select which FIT system(s) they wish to enroll

#### System Association

- Identify and create a FIT System record for each FIT system at a Site.

- FIT systems are associated with a Site, not a customer.

- Customers are granted access to the FIT system

### Initiate Authorization Process

Authorization Eligibility: Eligibility includes the following (Daikin can configure eligibility rules as needed):

- Site location

- Program availability

- FIT + Daikin One compatibility

#### Authorization Initiation: Initiate user authorization (includes below) within the SkyportEnergy app.

- Utility meter data access

- Program consent for load control

#### Authorization State: Track and display the authorization status (listed below) per FIT system. Time to complete authorization can vary from seconds to days given dependency on third parties (utility, aggregator, etc.).

- Not Started

- In Progress

- Authorized

- Failed

- Revoked/ Expired

### Resolve Authorization Conflicts with Customer

#### Authorization Diagnostics: Detect and classify authorization failures, including:

- Utility access revoked

- Incorrect service address

- Conflicting enrollments

- Insufficient permissions

- Processing delay

#### Customer-Facing Error Messaging: Display for each failure a clear explanation along with a recommended corrective action.

#### Reauthorization Flow: Enable reauthorization

- Without deleting the Site

- Without deleting or re-creating the FIT System

- Without re-pairing the One Thermostat

#### Disenrollment / Reset: Enable the customer to

- Disconnect a FIT system from an energy program

- Reset authorization for a specific FIT system

#### Support Escalation: Provide a “Contact Support” action that creates a support case and includes:

- Site ID

- FIT System ID

- Daikin One thermostat ID

- Authorization status & failure reason

- Timestamp of last attempt

# Phase #2: Configuration

Goal: Enable SkyportEnergy to confirm that authorized FIT systems are correctly configured for participation in energy programs by resolving enrollment conflicts and validating program nominations and meter configurations prior to dispatch.

Assumptions: Leap (aggregator) has

- Created or connected meters

- Configured nominations and meter groupings (these refer to logical, program-level configuration using existing utility revenue meters and do not require additional on-site metering hardware)

- Submitted meters to program operators

Enrollment Conflict Resolution:  Resolve enrollment conflicts for FIT systems during configuration (e.g., duplicate program enrollment, conflicting program participation, or invalid nominations).

Program Nomination Confirmation:

- Confirm that each authorized FIT system is nominated into the intended energy program(s) and aligns with required participation parameters (e.g., season, availability window)

- Validate program nominations against supported energy program definitions.

### Meter Configuration Validation

- Confirm that each participating FIT system is mapped to the appropriate utility meter(s) and aligns with the Site’s utility service configuration

- For Sites that contains multiple FIT systems, meter mappings may be shared, and validation shall still occur per FIT system

- Configuration issues shall be surfaced to the customer with clear guidance when corrective action is required.

Configuration Readiness State: Maintain a configuration status per FIT system (only below are considered dispatch-ready):

- Authorization is complete

- Enrollment conflicts are resolved

- Program nomination is confirmed

- Meter configuration is validated

# Phase #3: Dispatch

Goal: Execute authorized demand response events by receiving dispatch instructions, signaling participating customers, and controlling enrolled FIT systems in accordance with approved program parameters including allocation of fleet-level dispatch targets to individual FIT systems.

### Dispatch Intake

- Receive dispatch instructions from Leap for enrolled energy programs.

- Dispatch instructions shall reference eligible FIT systems and include event timing and participation parameters.

- Only FIT systems with a dispatch-ready configuration status shall be eligible to receive dispatch instructions.

### Dispatch Target Allocation

- Distribute the dispatch performance requirement received from Leap across eligible FIT systems based on dispatch-ready configuration and participation parameters.

- Allocate dispatch actions at the FIT system level to collectively meet the fleet-level dispatch requirement.

### Customer Dispatch Notification

- Issue dispatch signals to participating customers associated with eligible FIT systems.

- Dispatch signals shall reflect the active program event and expected participation window.

- Send system-specific notifications that are customer-specific and scoped to the affected Site.

### Load Curtailment

- Initiate load curtailment for participating FIT systems in accordance with program requirements during an active dispatch event.

- Dispatch actions shall be applied per FIT system and bounded by the authorized participation parameters.

- Track dispatch execution status per FIT system for the duration of the event.

### Dispatch Completion

- Return participating FIT systems to their pre-dispatch operating state after completion of a dispatch event.

- Dispatch outcomes shall be recorded per FIT system.

# Phase #4: Analyze

Goal: Evaluate dispatch execution and system performance to determine participation outcomes, verify compliance with program requirements, and support downstream settlement and reporting including evaluation of aggregated fleet-level performance.

### Dispatch Performance Evaluation

- Analyze dispatch execution per FIT system after each dispatch event to determine if the system responded as dispatched.

### Participation Validation

- Determine whether each participating FIT system met program participation requirements, accounting for system availability, authorization state, and dispatch execution status.

- Record validation results per FIT system.

### Underperformance Handling

- Record the underperformance outcome for dispatch events where participation requirements are not met.

### Aggregated Performance Evaluation

- Aggregate dispatch performance across participating FIT systems to evaluate whether the collective response meets the dispatch performance requirements received from Leap.

- Use aggregate performance results to support system-level attribution based on validated participation outcomes.

### Performance Attribution

- Attribute validated dispatch performance to the corresponding energy program and dispatch event.

### Analysis Output State

- Maintain an analysis state per FIT system and dispatch event.

- Indicate settlement eligibility for every dispatch event.

- Underperformance outcomes shall not be handled within the Analyze phase and may trigger configuration review or FIT system operational follow-up.

# Phase #5: Get Paid

Goal: Support settlement and payment processes by providing validated dispatch participation data to authorized program partners.

- Prepare settlement-eligible dispatch records per FIT system and event.

- Provide settlement data to authorized settlement partners.

- Track settlement status through delivery and acknowledgment.

- Surface informational payment outcome indicator to customers.